Student Name: Loren Hobbs
Course: Computer Networking
Assignment: Network Systems Design Proposal
Institution: Study.com
Date: May 14, 2026

Submitted Assignment

This proposal presents a secure and scalable network design for a new three-story office in Berlin, Germany. The design includes recommendations for switches, routers, wireless access points, security devices, physical media, and IP addressing. The goal is to support stable wired and wireless connectivity for Administration, Development, and Sales departments while allowing room for future growth.

Network Design Proposal for Berlin Office

Overview

The new Berlin office will support three floors: Administration on the first floor, Developers on the second floor, and Sales on the third floor. The network must support wired Ethernet devices, wireless users, VoIP phones, printers, guest access, and secure connections between users and company resources.

The proposed design uses 24-port Ethernet switches, Wi-Fi 6 access points, one edge router, a dedicated firewall, and structured cabling. Each floor will have its own access switches and wireless access points, with all floors connected back to a central network closet using fiber uplinks.

Task 1: Total Switches, Routers, and Wireless Access Points Needed

The office should use the following total network devices:

Network Design Proposal for Berlin Office

Overview

The new Berlin office will support three floors: Administration on the first floor, Developers on the second floor, and Sales on the third floor. The network must support wired Ethernet devices, wireless users, VoIP phones, printers, guest access, and secure connections between users and company resources.

The proposed design uses 24-port Ethernet switches, Wi-Fi 6 access points, one edge router, a dedicated firewall, and structured cabling. Each floor will have its own access switches and wireless access points, with all floors connected back to a central network closet using fiber uplinks.

Task 1: Total Switches, Routers, and Wireless Access Points Needed

The office should use the following total network devices:

| Device Type | Quantity | Justification |
| --- | --- | --- |
| Edge router | 1 | Provides routing between the Berlin office network and the internet/WAN connection. |
| Firewall/security gateway | 1 | Protects the internal network, supports VPN, filters traffic, and separates guest access from company traffic. |
| Core/distribution switch | 1 | Aggregates the floor switches and connects them to the router/firewall. |
| 24-port access switches | 7 | Provides wired connections for phones, laptops, printers, APs, and growth capacity. |
| Wi-Fi 6 access points | 7 | Supports wireless laptops, tablets, phones, and guest users with enough capacity for stable wireless service. |

The total design includes 8 switches if the core/distribution switch is counted, or 7 access switches if only floor-level access switches are counted.

The wireless demand is highest on the first floor, so that floor needs more AP capacity. Wi-Fi 6 is appropriate because it supports higher device density, better performance in busy environments, and improved handling of many simultaneous wireless users.

Task 2: Floor-Specific Device Allocation

First Floor: Administration

The first floor has the following network demand:

| Device Type | Count | Connection Type |
| --- | --- | --- |
| Printers | 2 | Ethernet |
| VoIP phones | 25 | Ethernet |
| Laptops | 20 | Ethernet and wireless |
| Tablets | 10 | Wireless |
| Cellphones | 20 | Wireless |
| Guest connections | 10 | Wireless |

The first floor has 47 wired devices if all laptops are also provided Ethernet connections:

2 printers + 25 VoIP phones + 20 laptops = 47 wired connections

Since each switch has 24 ports, two switches would technically provide 48 ports, but that leaves almost no room for uplinks, AP connections, spare ports, or future growth. Therefore, the first floor should use:

First floor recommendation: 3 switches and 3 Wi-Fi 6 APs

The 3 switches provide enough ports for all wired devices, AP connections, uplinks, and future expansion. The 3 APs support around 60 wireless users:

20 laptops + 10 tablets + 20 cellphones + 10 guest users = 60 wireless connections

Using 3 APs keeps the wireless load balanced and improves coverage across the floor.

Second Floor: Developers

The second floor has the following network demand:

| Device Type | Count | Connection Type |
| --- | --- | --- |
| Printers | 2 | Ethernet |
| VoIP phones | 20 | Ethernet |
| Laptops | 15 | Ethernet only |
| Tablets | 15 | Wireless |
| Cellphones | 15 | Wireless |
| Guest connections | 10 | Wireless |

The second floor has:

2 printers + 20 VoIP phones + 15 laptops = 37 wired connections

The wireless demand is:

15 tablets + 15 cellphones + 10 guest users = 40 wireless connections

Second floor recommendation: 2 switches and 2 Wi-Fi 6 APs

Two 24-port switches provide 48 ports, which is enough for the 37 wired devices, AP connections, uplinks, and limited growth. Two APs are appropriate because the floor has about 40 wireless users and needs stable coverage.

Third Floor: Sales

The third floor has the following network demand:

| Device Type | Count | Connection Type |
| --- | --- | --- |
| Printers | 2 | Ethernet |
| VoIP phones | 15 | Ethernet |
| Laptops | 10 | Ethernet and wireless |
| Tablets | 10 | Wireless |
| Cellphones | 10 | Wireless |
| Guest connections | 10 | Wireless |

The third floor has:

2 printers + 15 VoIP phones + 10 laptops = 27 wired connections

The wireless demand is:

10 laptops + 10 tablets + 10 cellphones + 10 guest users = 40 wireless connections

Third floor recommendation: 2 switches and 2 Wi-Fi 6 APs

Two switches provide 48 Ethernet ports, which is enough for wired devices, APs, uplinks, and spare capacity. Two APs provide better coverage and load balancing than a single AP.

Floor Allocation Summary

| Floor | Wired Devices | Wireless Devices | Switches Needed | APs Needed |
| --- | --- | --- | --- | --- |
| 1st Floor: Administration | 47 | 60 | 3 | 3 |
| 2nd Floor: Developers | 37 | 40 | 2 | 2 |
| 3rd Floor: Sales | 27 | 40 | 2 | 2 |
| Total | 111 | 140 | 7 access switches | 7 APs |

The total number of endpoint devices is:

87 first-floor devices + 77 second-floor devices + 57 third-floor devices = 221 endpoint devices

This does not include infrastructure devices such as APs, switches, router interfaces, management interfaces, or future growth. Because of that, the network should not be designed with only the exact number of required addresses or ports.

Task 3: Security Devices and Justification

The office should include the following security devices and services:

Firewall / Security Gateway

A dedicated firewall should be placed between the internal office network and the internet connection. The firewall should inspect traffic, block unauthorized access, and enforce security rules between internal departments and external networks.

The firewall should also support VPN access for remote employees. This allows employees to securely connect to company resources when working outside the office.

VPN Device or Firewall-Based VPN

The company should use a VPN, either built into the firewall or provided through a dedicated VPN appliance. This is important for remote developers, administrators, and sales staff who may need secure access to internal systems.

VLAN-Capable Switches

The switches should support VLANs. VLANs allow the company to separate traffic by department and device type. For example, the network should have separate VLANs for:

| VLAN | Purpose |
| --- | --- |
| Administration VLAN | First-floor employee devices |
| Developer VLAN | Second-floor employee devices |
| Sales VLAN | Third-floor employee devices |
| Voice VLAN | VoIP phones |
| Printer VLAN | Shared printers |
| Guest VLAN | Guest wireless traffic |
| Management VLAN | Switches, APs, and network administration |

This improves performance and security because guest users should not be on the same network as company laptops, phones, or servers.

Wireless Security

The Wi-Fi network should use WPA3 security for company wireless access. Guest Wi-Fi should be isolated from the internal network and should only allow internet access. This protects company resources if a guest device is infected or untrusted.

Authentication Server

The company should use centralized authentication, such as RADIUS or 802.1X, for employee Wi-Fi and wired network access. This makes it harder for unauthorized users to connect to the network just by plugging into an Ethernet jack or knowing a shared Wi-Fi password.

Task 4: Physical Media Selection

The recommended physical media is a combination of copper Ethernet cabling and fiber optic cabling.

Cat6 or Cat6a Ethernet Cable

Cat6 or Cat6a cable should be used for connections from wall outlets to endpoint devices such as laptops, printers, VoIP phones, and wireless access points.

Cat6 is appropriate because it supports high-speed Ethernet and is reliable for normal office cable distances. Cat6a is also a good choice if the company wants better support for 10 Gigabit Ethernet and future upgrades.

Fiber Optic Backbone

Fiber optic cable should be used between the floor network closets and the main network closet. This is the best choice for inter-floor connections because fiber supports high bandwidth, longer distances, and better resistance to electrical interference.

Patch Panels and Structured Cabling

Each floor should have a small network closet or telecommunications cabinet with patch panels. The patch panels make the cabling easier to manage, troubleshoot, and expand. Structured cabling also makes future moves, adds, and changes easier.

Power over Ethernet

The access switches should support Power over Ethernet, also called PoE. PoE allows the switches to provide power to VoIP phones and wireless access points over the Ethernet cable. This reduces the need for separate power adapters and makes AP placement easier.

Task 5: IP Address Class and Subnet Mask

The Berlin office has 221 endpoint devices before counting network infrastructure, switch management interfaces, Aps, router/firewall interfaces, and future growth. A single Class C network using a /24 subnet would provide only 254 usable host addresses. That would technically fit the current endpoint count, but it would leave very little room for growth and would not be ideal for VLAN separation.

A better choice is to use a private Class A address range, such as:

10.20.0.0/16

This private address range provides enough room for multiple VLANs, future growth, and separate subnets for each department and device type.

A practical subnet plan would use /24 subnets, with a subnet mask of:

255.255.255.0

Each /24 subnet provides 254 usable host addresses, which is enough for each floor or VLAN.

Example subnet plan:

| Network Segment | Subnet | Subnet Mask | Usable Hosts |
| --- | --- | --- | --- |
| Administration | 10.20.10.0/24 | 255.255.255.0 | 254 |
| Developers | 10.20.20.0/24 | 255.255.255.0 | 254 |
| Sales | 10.20.30.0/24 | 255.255.255.0 | 254 |
| Voice | 10.20.40.0/24 | 255.255.255.0 | 254 |
| Printers | 10.20.50.0/24 | 255.255.255.0 | 254 |
| Guest Wi-Fi | 10.20.60.0/24 | 255.255.255.0 | 254 |
| Network Management | 10.20.70.0/24 | 255.255.255.0 | 254 |

This design is more scalable than putting all devices into one subnet. It also improves security because firewall and routing rules can control traffic between VLANs. For example, guest Wi-Fi users can be allowed to reach the internet but blocked from reaching company laptops, printers, phones, and internal systems.

Conclusion

The proposed Berlin office network uses 7 access switches, 1 core/distribution switch, 1 edge router, 1 firewall/security gateway, and 7 Wi-Fi 6 access points. This design provides enough wired and wireless capacity for all three floors while leaving room for growth.

The design also improves security by using VLANs, firewall policies, VPN access, WPA3 wireless security, guest network isolation, and centralized authentication. Cat6 or Cat6a cabling should be used for endpoint connections, while fiber optic cabling should be used for the building backbone between floors.

For IP addressing, the best choice is a private Class A address range, such as 10.20.0.0/16, divided into /24 subnets using the subnet mask 255.255.255.0. This gives the company a stable, secure, and scalable network design for its new European headquarters.